CECS 346 Project 1

Traffic Light Controller with Moore FSM

© 2026 Dr. Min He, CSULB. All rights reserved.

# Overview

Design and build a traffic light controller for a 4-corner intersection of two one-way streets (South and West) with a pedestrian crossing. Your system uses a Moore FSM with SysTick timer to coordinate three participants: South traffic, West traffic, and pedestrians. This project integrates all skills from Labs 1–4: GPIO initialization, FSM design, struct-based state tables, SysTick timing, and hardware interfacing.

## Learning Objectives

- Design a Moore FSM with 9 states, 3 inputs (8 combinations), and multiple outputs

- Implement the FSM using an indexed data structure (struct array) in C

- Use SysTick timer for all timing delays (no software delay loops)

- Interface multiple GPIO ports: Port B (traffic LEDs), Port E (sensors), Port F (pedestrian LEDs)

- Use bit-specific addressing for all I/O access

- Build and debug a multi-port embedded system on a breadboard

- Apply round-robin fairness scheduling for competing intersection participants

## Preparation

Complete before starting:

- Read textbook Sections 6.4–6.9

- Review reference projects: SimpleTrafficLight, SysTick

- Have a working Lab 4 project (starter project for this project)

Materials needed:

- TM4C123 LaunchPad

- 6 LEDs: 2 red, 2 yellow, 2 green (for South and West traffic lights)

- 3 switches: preferably slide switches or DIP switches (not momentary push buttons)

- Resistors for LED current limiting (330Ω–1kΩ)

- Breadboard and jumper wires

Note: Slide switches or DIP switches are strongly preferred because sensors must remain ON until the FSM reads them. Push buttons require holding throughout the entire state duration.

# System Requirements

Design a traffic light controller for the intersection of two equally busy one-way streets. The goal is to maximize traffic flow, minimize waiting time, and avoid accidents.

Figure 1: 4-corner intersection with South, West, and Pedestrian participants

## Three Participants

| Participant | Sensor | Green Lights | Logic |
| --- | --- | --- | --- |
| South | PE2 — car sensor | PB0 (G), PB1 (Y), PB2 (R) | Positive: 3.3V = car present |
| West | PE1 — car sensor | PB3 (G), PB4 (Y), PB5 (R) | Positive: 3.3V = car present |
| Pedestrian | PE0 — walk button | PF3 (walk), PF1 (don't walk) | Positive: 3.3V = ped waiting |

## Pedestrian Light Sequence

| Phase | LED | Duration | Meaning |
| --- | --- | --- | --- |
| Walk | PF3 green ON | 2 sec | Pedestrians may cross |
| Hurry Up | PF1 red FLASHING | 1 sec total | Finish crossing quickly |
| Don't Walk | PF1 red ON steady | — | Do not enter intersection |

Hurry Up flashing: 0.25s on, 0.25s off, 0.25s on, 0.25s off = 1 second total (4 states).

## Timing Summary

| Phase | Duration |
| --- | --- |
| Green / Walk | 2 seconds (8 × 0.25s) |
| Yellow / Hurry | 1 second (4 × 0.25s) |

Base time unit: 0.25 seconds. Use SysTick timer (not software delay loops).

# Fairness Rules (Round-Robin Scheduling)

When multiple participants need the intersection, service them fairly using these rules:

Rule 1: Traffic lights must follow green → yellow → red. No other transition is allowed.

Rule 2: Pedestrian lights must follow walk → hurry → don't walk. No other sequence is allowed.

Rule 3: Once a participant starts a green-yellow-red (or walk-hurry-don't walk) transition, it must complete the full sequence before giving green to another participant.

Rule 4: When two participants compete for green after a transition, neither may be the one that just had green. Alternate fairly between the two competitors.

Example (Rule 4): After GoS (South had green), if both West and Pedestrian need green, assign one in case 1 (all three competing) and the other in case 2 (only two competing). This ensures fair alternation.

The system starts with Green on South.

# Implementation Requirements

⚠ No conditional branches (if-else) in the FSM engine. No #include tm4c123gh6pm.h — you must define all registers yourself.

- Implement a Moore FSM: each state has a name, output, wait time, and 8 next states

- Use SysTick timer for all delays

- Use bit-specific addressing for P_LIGHTS, T_LIGHTS, and SENSORS

- Do not embed functionality (e.g., flash twice) into the engine that is not in the state graph

## GPIO Pin Assignments

| Traffic Light | Pin |
| --- | --- |
| Green south | PB0 |
| Yellow south | PB1 |
| Red south | PB2 |
| Green west | PB3 |
| Yellow west | PB4 |
| Red west | PB5 |

| Sensor / Button | Pin |
| --- | --- |
| South car sensor | PE2 |
| West car sensor | PE1 |
| Pedestrian Walk | PE0 |
| Walk light (green) | PF3 |
| Don't Walk light (red) | PF1 |

## Required Bit-Specific Address Definitions

#define P_LIGHTS  (*(volatile uint32_t *)0x_________)  // PF3, PF1

#define T_LIGHTS  (*(volatile uint32_t *)0x_________)  // PB5-PB0

#define SENSORS   (*(volatile uint32_t *)0x_________)  // PE2-PE0

You must calculate these addresses using base + (bit_mask << 2).

# FSM Design

## State List (9 states)

| State | Time (×0.25s) | PB5-0 Output | PF3,1 | Description |
| --- | --- | --- | --- | --- |
| GoS | 8 | 100001 | 01 | South green, West red, Don't walk |
| WaitS | 4 | (fill in) | (fill in) | South yellow, West red, Don't walk |
| GoW | 8 | (fill in) | (fill in) | South red, West green, Don't walk |
| WaitW | 4 | (fill in) | (fill in) | South red, West yellow, Don't walk |
| GoP | 8 | (fill in) | (fill in) | Both red, Walk (green on) |
| WaitPOn1 | 1 | (fill in) | (fill in) | Both red, Hurry flash ON (1st) |
| WaitPOff1 | 1 | (fill in) | (fill in) | Both red, Hurry flash OFF (1st) |
| WaitPOn2 | 1 | (fill in) | (fill in) | Both red, Hurry flash ON (2nd) |
| WaitPOff2 | 1 | (fill in) | (fill in) | Both red, Hurry flash OFF (2nd) |

## State Transition Table (you complete this)

Input order: PE2 (South), PE1 (West), PE0 (Pedestrian) → 3 bits → 8 combinations (000–111).

Partially filled — complete the remaining cells using the fairness rules:

| State | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GoS | GoS | WaitS | WaitS | WaitS | GoS | WaitS | WaitS | WaitS |
| WaitS | GoW | GoP | GoW | GoW | GoP | GoP | GoW | GoP |
| GoW |  |  |  |  |  |  |  |  |
| WaitW |  |  |  |  |  |  |  |  |
| GoP | GoP | GoP | WaitPOn1 |  |  |  |  |  |
| WaitPOn1 | WaitPOff1 | WaitPOff1 | WaitPOff1 | WaitPOff1 | WaitPOff1 | WaitPOff1 | WaitPOff1 | WaitPOff1 |
| WaitPOff1 | WaitPOn2 | WaitPOn2 | WaitPOn2 | WaitPOn2 | WaitPOn2 | WaitPOn2 | WaitPOn2 | WaitPOn2 |
| WaitPOn2 | WaitPOff2 | WaitPOff2 | WaitPOff2 | WaitPOff2 | WaitPOff2 | WaitPOff2 | WaitPOff2 | WaitPOff2 |
| WaitPOff2 |  |  |  |  |  |  |  |  |

# Implementation Steps

Step 1: Design the FSM. Draw the state diagram showing all 9 states, outputs, wait times, and transitions. Complete the state table above.

Step 2: Write and debug in Keil simulator. Use the Logic Analyzer to verify all input/output combinations.

Step 3: Build the switch circuits (positive logic). Wire 3 switches to PE2, PE1, PE0. Test with a simple program that maps each switch to an onboard LED.

Step 4: Build the LED circuits (positive logic). Wire 6 LEDs to PB5–PB0 arranged like a traffic intersection. Test with a program that turns all LEDs on then off.

Step 5: Integrate and debug the complete system on the LaunchPad.

⚠ Do not place or remove wires on the breadboard while power is on.

# Onboard Demonstration Steps

Follow this sequence for the live demo:

1. Reset: South green on, West red, Pedestrian don't walk (red steady).

2. Single participant tests:

  - Turn on West sensor → observe South green→yellow→red, then West green

  - Turn on Pedestrian → observe West green→yellow→red, then Walk green

  - Turn on South → observe Walk→Hurry→Don't Walk, then South green

  - Turn on Pedestrian → Walk sequence, then back to another participant

  - Turn on West → transition to West green

  - Turn on South → transition to South green

3. Two participant tests (observe two full rounds):

  - South + Pedestrian: hold both, watch alternating service

  - South + West: hold both, watch alternating service

  - West + Pedestrian: hold both, watch alternating service

4. Three participant test: Hold all three, observe two full rounds of round-robin cycling.

# Deliverables and Grading

| # | Item | Format | Points | Due |
| --- | --- | --- | --- | --- |
| 1 | On-board demonstration (all test cases). All team members present. | Live demo | 30 | In lab |
| 2 | Logic Analyzer simulation screenshots (test case 4: all three participants) | In report | 10 | Report |
| 3 | Video or link for on-board demo (all test cases) | Link in report | 10 | Report |
| 4 | Source code (.c/.h files). No startup.s, no tm4c123gh6pm.h. Define all registers. | Separate files | 20 | Report |
| 5 | Project report (see format below) | Word or PDF | 30 | Report |

## Project Report Format

Hardware Design: Professional schematic and photo of your embedded system.

Operation: Include the video link for on-board demonstration.

Software Design: State table, state diagram, and list of .c/.h source files.

Conclusion: Challenges, how you solved them, and what you learned most from this project.